SodaStream Pure

Communication Protocol Between the PC Software

and the Embedded Software on the Board

Last Update: 9/12/2025

Updated by: Yossi CS

Updates History:

| Date | By | Details |
| --- | --- | --- |
| 21/5/2025 | Yossi CS | Initial creation |
| 8/6/2025 | Yossi CS | English version |
| 22/6/2025 | Yossi CS | Updated following review |
| 22/6/2025 | Yossi CS | Some typos and minor corrections |
| 26/06/2025 | Roy S | RGB LEDS range. |
| 5/8/2025 | Yossi CS | Accelerometer values |
| 8/8/2025 | Yossi CS | Added command to set carbonation table, updated RSTS command the response. |
| 11/8/2025 | Yossi CS | Added the 0.5L rows to the carbonation table. Added CONF command. Updated: SRGB and SLED |
| 12/8/2025 | Yossi CS | Added the SWTS command |
| 13/8/2025 | Yossi CS | Added the LPTM command |
| 15/8/2025 | Yossi CS | Added 3rd config parameter and changed to 32 bits |
| 27/10/2025 | Yossi CS | RSTS – RTC returns days and seconds since last filter reset (instead of seconds) |
| 18/11/2025 | Yossi CS | Added the FDAY, CMAX and CSEC commands |
| 9/12/2025 | Yossi CS | Added the VBAT command, removed the SRGB command, added table of contents. |

Table of Contents

General	3

Protocol and Command Definition	4

Command List	5

1.	Command: MGUI S	5

2.	Command: POWR S	6

3.	Command: 1UVLD S	6

4.	Command: SLED L,P	7

5.	Command: PUMP S,W	8

6.	Command: CARB S	8

7.	Command: STOP	9

8.	Command: DONE status	9

9.	Command: TILT	10

10.	Command: WTRS	11

11.	Command: UVLA	12

12.	Command: PMPA	13

13.	Command: RRTC	14

14.	Command: RSTS M,I	15

15.	Command: RSTS mask,status	16

16.	Command: RVER	17

17.	Command: STBL <Row- ID>,<val1>,<val2>,<val3>,<val4>,<val5>, <val6>,<val7>,<val8>	18

18.	Command: SWSP S	20

19.	Command: LPTM <millisecs>	21

20.	Command: FDAY <days counter>,<max days>,<warning period>	22

21.	Command: CSEC S,T	23

22.	Command: VBAT	23

# General

The embedded system has two modes of operation:

- Automatic Mode – standalone button-activated

  - The default mode after power-up.

  - The device is operated using its physical buttons.

  - Functions autonomously — the GUI may or may not be connected. In this mode, the device only responds to the command to switch to GUI Mode (MGUI 1) or to non-intrusive information requests (e.g., reading the device version)

  - The device transmits general status messages over UART, such as version information on power-up and "Process Completed" (DONE 0000) when filtering and/or carbonation are finished. These messages allow state detection by the GUI (if connected). For details on the messages transmitted in this mode, see below.

- GUI-Controlled Mode – Controlled via UART by an External Windows GUI Application

  - The device enters this mode only after receiving the MGUI command.

  - This mode resets to Automatic Mode upon a power cycle.

  - The GUI has full control over the device, including starting/stopping peripherals. However, the device remains independently responsible for time-critical event detection and reactions (such as stopping the pump when sensor detects water – if requested).

  - Physical button presses on the device are ignored in this mode.

# Protocol and Command Definition

- The protocol is text-based.

- Each message starts with the $ symbol, followed by a fixed-length 4-character command.

- If the command has parameters, they begin after a space, and multiple parameters are separated by commas.

- Each command ends with <CR><LF> (i.e., the string \r\n).

- Every command sent from the PC to the board will receive an ECHO of the command as confirmation of receipt (i.e., the board sends back to the PC the exact same command and parameters it received), even if the command was sent at an invalid time.

- For the commands RVER, there is no ECHO – a response is received immediately. For STBL – see below.

- Invalid commands (e.g., unrecognized, incorrect number of parameters, etc.) will receive the response: $Illegal Command.

- All numeric parameters are limited to 16 bits (0-65535)

- The interface operates at 115200,8,N,1 (115200 baud rate, 8 data bits, No parity, One stop bit).

# Command List

## Command: MGUI S

Direction: PC to Embedded.

Parameters:

S – state 0 – off, 1 – on

Echo: Yes

Set the device mode: MGUI 1 – GUI Controlled mode, MGUI 0 - Automatic mode.

Example:

$MGUI 1

## Command: POWR S

Direction: PC to Embedded.

Parameters:

- S – state 0 – off, 1 – on

Echo: Yes

Description:

Turn On or Off the power signal of the Solenoid/Pump board.

Example:

$UVLD 1

## Command: 1UVLD S

Direction: PC to Embedded.

Parameters:

- S – state 0 – off, 1 – on

Echo: Yes

Description:

Turn On or Off the UV LED.

Example:

$UVLD 1

## Command: SLED L,P

Direction: PC to Embedded.

Parameters:

- L – Led number: TBD (will be updated on the Pure-VDL)

  - 1 – Carbonation level 1 led

  - 2 – Carbonation level 2 led

  - 3 – Carbonation level 3 led

  - 4 – Led next to battery

  - 5 – Led next to battery

- P – percent value 0-100 – intensity of the led (0=off to 100=on)

Echo: Yes

Description: Set an LED according to the percentage provided.

Example:

$BTLD 1,50

## Command: PUMP S,W

Direction: PC to Embedded.

Parameters:

- S – state 0 – off, 1 – on

- W – 0 – ignore water sensor, N – automatically stop on sensor value above N

Echo: Yes

Description: Turn On or Off the water pump. If the “With sensor” specified, the pump will automatically stop when the sensor detects water – i.e. went above the threshold value N (0-65535).  When N > 0, the sensor should be enabled using the SWSP command.

See the WTRS command below – that command enables to calibrate the sensor value.

The second parameter is ignored when setting off.

N default value is 500.

Example:

$PUMP 1,1234

## Command: CARB S

Direction: PC to Embedded.

Parameters:

- S – state 0 – off, 1 – on

Echo: Yes

Description:

Turn On or Off the pump + solenoid.

Example:

$CARB 1

## Command: STOP

Direction: PC to Embedded.

Parameters: none

Echo: Yes

Description: Stop whatever process is running as activated by the PUMP or by CARB commands and turn off whatever is activated by the command.

Example:

$STOP

## Command: DONE status

Direction: Embedded to PC

Parameters: 4 digits number indicating the done status

0000 – In automatic mode: The process (filter, rinse, carb, filter + carbonation) started by last manual key press completed OK. In GUI mode: the water pump has stopped after sensor detected water (if PUMP 1,1 command is issued)

0050 – In GUI mode: Process interrupted due to STOP or any other command stopping command

0101 – In any mode: Water Pump failed

0111 – In any mode: UV Error

0121 – In any mode: Solenoid Error

Description:

Carbonation done message with a parameter indicating the completion reason.

Example:

$DONE 0000

## Command: TILT

Direction: PC to Embedded.

Parameters: none

Description: Read Tilt state

Example:

$TILT

Response: TILT X,Y,Z

Direction: Embedded to PC.

Parameters:

- X –X acceleration

- Y – Y acceleration

- Z – Z acceleration

Description: The response is a comma-separated string containing three signed 8-bit integers representing time-filtered acceleration values along the X, Y, and Z axes at the moment the response is sent. These values are scaled to ±2g range, filtered using a weighted moving average, and include gravitational acceleration.

To convert it to actual angles in degrees, use this formula (

θa= tan-1AaAb2+Ac2×180π
where:
θa – The angle of axis a

Aa – Axis a acceleration in g – the number in the message X 0.0156

Ab,Ac – The other two axis acceleration in g – the numbers in the message X 0.0156

Example:

$TILT 23,-3,195

The actual angles: 6.73°, –0.88°, 83.22°

## Command: WTRS

Direction: PC to Embedded.

Parameters: none

Description: Read water level sensor value

The SWSP needed to be called first to enable the water sensor.

Example:

$WTRS

Response: WTRS L

Direction: Embedded to PC.

Parameters:

- L – read value

Description: Response to WTRS command

Example:

$WTRS 12345

## Command: UVLA

Direction: PC to Embedded.

Parameters: none

Description: Read UV led current

Example:

$UVLA

Response: UVLA C

Direction: Embedded to PC.

Parameters:

- C – current value

Description: Response to UVLA command

Example:

$UVLA 1234

## Command: PMPA

Direction: PC to Embedded.

Parameters: none

Description: Read pump current

Example:

$PMPA

Response: PMPA C

Direction: Embedded to PC.

Parameters:

- C – current value

Description: Response to PMPA command

Example:

$PMPA 1234

## Command: RRTC

Direction: PC to Embedded.

Parameters: none

Description: Read the RTC value

Example:

$RRTC

Response: RRTC S

Direction: Embedded to PC.

Parameters:

- D – total days since last filter reset

- S – total seconds since last filter reset

Description: Response to RRTC command

Example:

$RRTC 12,1080935

## Command: RSTS M,I

Direction: PC to Embedded.

Parameters:

- M – Mask of requested status messages

  - 1 – state and buttons

  - 2 – ADC values

  - 4 – RTC and Tilt

- I – interval in milliseconds (N >= 100) ignored if mask is 0

Description: Request to start/stop periodic status messages transmission each M milliseconds. Each bit set in the mask will result in an additional status message sent at the period of the interval.

Sending a 0/1 in any of bit mask will stop/start sending the information related to the bit e.g. 7 (1+2+4) will send 1,2,4 related messages, 5 (1+4) will stop sending the ADC values.

Sending 0 as the entire mask will stop all messages

Status messages are sent only in GUI Mode.

Example:

$RSTS 500

## Command: RSTS mask,status

Direction: Embedded to PC.

Parameters:

- Mask – message bit mask (1,2,4,or 8)

- status – comma delimited values

For 1 - state and buttons:

<state> - 1 – Gui Controlled mode OK, 11 – tilt error, 12 – HW watchdog error, 13 – Safety failed error

<Filter Button> - 0 – released, 1 – pressed

<Low Button> - 0 – released, 1 – pressed

<Med Button> - 0 – released, 1 – pressed

<High Button> - 0 – released, 1 – pressed

For 2 – ADC value:

<Water Level sensor ADC value>

<UV Led ADC value>

<Pump ADC value>

For 4 - RTC and Tilt

<RRTC> - as in RRTC – total days since last filter reset

<Tilt X> - X Acceleration – see TILT command

<Tilt Y> - Y Acceleration – see TILT command

<Tilt Z> - Z Acceleration – see TILT command

Description: The status messages will be sent one by one each interval

Example:

$RSTS 1,1,1,0,0,0
$RSTS 2,1230,1734,1300

## Command: RVER

Direction: PC to Embedded.

Parameters: none

Description: Read the embedded software version number.

Example:

$RVER

Response: RVER <version string>

Direction: Embedded to PC

Parameters: A string of the version number

Description: A response to the RVER command, sent also independently upon power up.

Example:

$RVER version 1.2.0

## Command: STBL <Row- ID>,<val1>,<val2>,<val3>,<val4>,<val5>, <val6>,<val7>,<val8>

Direction: PC to Embedded.

Parameters:

- <Row ID> -

1 – 1 L, Low level table Ton

2 – 1 L, Low level table Toff

3 – 1 L, Medium level table Ton

4 – 1 L, Medium level table Toff

5 – 1 L, High level table Ton

6 – 1 L, High level table Toff

7 – 0.5 L, Low level table Ton

8 – 0.5 L, Low level table Toff

9 – 0.5 L, Medium level table Ton

10 – 0.5 L, Medium level table Toff

11 – 0.5 L, High level table Ton

12 – 0.5 L, High level table Toff

- <Val X> -  Column X time value in milliseconds of the row

Description: Sets the carbonation time tables for the Pump and Valve at each level up to 8 periods. The time values in milliseconds are limited to 16 bits (0-65535)

Note that the sequence will end on the first 0 Value of “On” (or the 8th)

If the user inserted a bottle and selected carbonation without filling the bottle using the pump, the 1L bottle size settings are used.
The default table after power up:

| Bottle | Level | Action | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 L | Low | On | 700 | 1000 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 L | Low | Off | 500 | 500 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 L | Medium | On | 700 | 1000 | 1000 | 0 | 0 | 0 | 0 | 0 |
| 1 L | Medium | Off | 500 | 500 | 500 | 0 | 0 | 0 | 0 | 0 |
| 1 L | High | On | 700 | 1000 | 1000 | 1250 | 1250 | 0 | 0 | 0 |
| 1 L | High | Off | 500 | 500 | 500 | 500 | 500 | 0 | 0 | 0 |
| 0.5 L | Low | On | 700 | 1000 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0.5 L | Low | Off | 500 | 500 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0.5 L | Medium | On | 700 | 1000 | 1000 | 0 | 0 | 0 | 0 | 0 |
| 0.5 L | Medium | Off | 500 | 500 | 500 | 0 | 0 | 0 | 0 | 0 |
| 0.5 L | High | On | 700 | 1000 | 1000 | 1250 | 1250 | 0 | 0 | 0 |
| 0.5 L | High | Off | 500 | 500 | 500 | 500 | 500 | 0 | 0 | 0 |

Example – sets the first row in the table:

$STBL 1,700,1000,800,0,0,0,0,0

Response: STBL <Row- ID>,OK

Direction: Embedded to PC

Parameters:

- <Row ID> - the received row ID

- “OK” – got it OK (Note that invalid data will return “Illegal command” as with other commands)

Description: An acknowledge for setting a row.

Example:

$STBL 1,OK

Response: CONF <Param ID>,<Value>

Direction: PC to Embedded.

Parameters:

- <Param ID> - the ID of the parameter to configure, where:

  - 1 – Threshold in milliseconds of pump time to differ 1 litter bottle from 0.5 litter bottle.

  - 2 – Timeout in milliseconds of pump time. Stops the pump in case water level sensor has not passed the threshold - as last set by W parameter of the PUMP command. This parameter does not apply to GUI control mode.

  - 3 – Water sensor threshold value.

- <Value> – Value to set

Description: Set configuration parameter.

Example:

$CONF 1,10

## Command: SWSP S

Direction: PC to Embedded.

Parameters:

- S – state 0 – off, 1 – on

Description: Set water sensor power on or off.

The command can used only in GUI mode. If turning the pump on using the PUMP command with threshold value for the sensor, this command needed to be called first.

Example:

$SWSP 1

Response: LPTM

Direction: PC to Embedded.

Parameters: None

Description: Request that last pump working period.

Example:

$LPTM

## Command: LPTM <millisecs>

Direction: Embedded to PC

Parameters:

- <millisecs> - The period in milliseconds

Description: Response to LPTM command.

The value returned is measured if pump stopped automatically on water sensor, by the command PUMP 0,x or by timeout. The value is reset each time the pump is starting.

Example:

$LPTM 15678

Response: FDAY S,D

Direction: PC to Embedded.

Parameters:

- S – 0 - Read, 1 - Write

- D – Number of days (ignored for S=0), valid values: 0-200

Echo: Yes

Description: Reads or sets the days counter of the filter. Note that the days counter is progressing automatically by the RTC and kept running by the backup battery while the machine is disconnected from power, the purpose of the “set” in this command is for debugging.
This command can be executed in any mode.

Example:

$FDAY 1,50

## Command: FDAY <days counter>,<max days>,<warning period>

Direction: Embedded to PC

Parameters:

- <days counter> - The current value of the filter days counter starting from 0 and growing up to <max days> i.e. setting to a number greater than maximum will show the maximum value.

- <max days> - The maximum lifetime of a filter in days

- <warning period> - The time in days, before getting to maximum, to show filter replacement warning. i.e. <days counter> is greater than <max days> minus <warning period> and still smaller than <max days>

Description: Response to FDAY command.

The values return – as described above.

This command can be executed in any mode.

Example:

$FDAY 5,90,9

Response: CMAX T

Direction: PC to Embedded.

Parameters:

- T – Number of seconds

Echo: Yes

Description: Sets the CO2 usage maximum seconds counter. This value is persistent in the RTC backup memory therefore it is saved between device power cycles.

This command can be executed in any mode.

Example:

$SMAX 170

## Command: CSEC S,T

Direction: PC to Embedded.

Parameters:

- S – 0 - Read, 1 - Write

- T – Seconds (ignored for S=1)

Echo: Yes

Description: Reads or sets the current value of the CO2 seconds counter. The purpose of the “set” in this command is for debugging.
This command can be executed in any mode.

Example:

$FDAY 1,50

Response: CSEC <CO2 seconds counter>,<max counter value >

Direction: Embedded to PC

Parameters:

- < CO2 seconds counter > - The current value of the CO2 usage maximum seconds conter.

- <max counter value> - The maximum counter value as set by default (170) or by the CMAX command.

Description: Response to “FDAY 0,0” command.

The values return – as described above.

This command can be executed in any mode.

Example:

$CSEC 43,170

## Command: VBAT

Direction: PC to Embedded

Parameters: none

Description: Read last VBAT value (ADC).

This command can be executed in any mode.

Example:

$VBAT

Response: VBAT <VBAT ADC value >

Direction: Embedded to PC

Parameters:

- <VBAT ADC value> - The current ADC value of the VBA.

Description: Response to “VBAT” command.

The values return – as described above.

This command can be executed in any mode.

Example:

$VBAT 1463